%N bee

%D Fit polynomials and antenna positions to the antenna gains.

%P mchw

%: uv analysis

%B

BEE is a Miriad task to fit polynomials and antenna positions

to the antenna gains. The gains can be plotted versus time, HA,

DEC, elevation, etc, and identified by source. Polynomial or

cursor drawn fits to the amplitude and phase can be written as

antenna gains and used to calibrate the data.

%A vis

The input UV dataset name. The antenna gains must first be

derived for the selected linetype by the task SELFCAL with

options=apriori,amplitude,noscale minants=3 interval=inttime

in order to fit Jy/K to individual integrations. No default.

%A log

The output log file. The default is 'bee.log'.

%A device

PGPLOT diplay device. (e.g. ? /tek /retro /sun /xw) No default.

Hardcopy plots can be created using cursor options.

%A refant

The gain of this antenna is set to cmplx(1.,0.). The

other antenna gains are relative to the reference antenna.

The default is to use the original reference antenna used to

derive the antenna gains. If refant.ne.0 then the total power

for the refant is subtracted from each of the others, and can

be plotted over the antenna phases to look for correlations.

%A refpwr

Put tpower for this reference antenna into the airtemp variable

in order to fit phase = a*tpower + b*tpower(refpwr) + c

using the ``TF'' 3-parameter fit

%A out

The output gains file. The fits to the amplitude and phase

can be written as antenna gains and used to calibrate the data.

The default is to write the gains into the input uvdata file.

%A options

rescale      The total powers are scaled to the same range as

the reference antenna. The default is no scaling.