%N uvcheck

%D Check uv-variable values, etc and flag uv-data.

%P mchw

%: uv analysis, checking, flagging

%B

UVCHECK is a Miriad program to check uv-variable values

and flag uv-data if variables or data are out of range.

The default is to list the data

when the source, freq, or number of channels change.

The number of records, uvrange and time range are printed.

The statistics for a selected uv-variable can

be printed. uv-data can be flagged if the uv-variable,

the amplitude of a reference channel, the geometric delay,

or the fringe frequency is outside a specified  range.

uvcheck does not handle systemp correctly unless you select

uvdata by antenna and line.

%A vis

The input visibility file. No default.

%A select

This selects which visibilities to be used. Default is

all visibilities. See the Users Guide for information about

how to specify uv-data selection.

%A line

Linetype of the data in the format line,nchan,start,width,step

"line" must be either `channel' or `wide'  for flagging data.

The default is all the channel data, and the wideband data.

Use line=wide to check only the wideband data.

%A ref

Reference line for flagging data.

Allowed values are:

channel,start,width

wide,start,width

Default: no reference line.

%A refamp

Minimum and maximum values for the reference line amplitude.

Flag the uv-data when the reference amplitude

is OUTSIDE the selected range.

The flags are set to the value given by flagval.

Calibration files (gains and passband) are NOT applied.

Default: 0,1e20    i.e. don't flag any uv-data.

%A flagval

set to either 'flag' or 'unflag' to flag the data.

Default: don't change the flags.

%A var

Name of uv-variable to check. Default is none.

Print mean and rms values of variable WITHIN the selected range.

A histogram plot can be printed (options=histo).

Special variables calculated from other uv-variables:

1) var=uvdist, where uvdist is in nanosecs.

2) var=fringe, where fringe is the fringe frequency in Hz.

%A range

Minimum and maximum values for uv-variable.

Flag the uv-data when the uv-variable is OUTSIDE this range.

Default range=-1E20,1E20. Note that in in order to flag

data within a selected range, first flag everything, then

unflag those outside the selected range.

%A delayflag

Flag the uv-data for each baseline if the absolute value of the

difference in the delay lines is less than the specified value.

Default delayflag=0.d0  does not flag any uv-data.

%A log

The output log file. Default is the terminal.

%A options

Extra processing options. Possible values are:

debug    Print more output

histo    Print histogram