%N gplist

%D List gains table and optionally overwrite calibration

%P smw

%: calibration

%B

GpList is a Miriad task to list the amplitude gains in a gains table

and (optionally) replace the amplitude information in a gains

table with constants of the users choosing, and/or simultaneously

setting the phase corrections to zero.

WARNING: if you modify the gains, the resulting amplitude gains will be

constant with time! The original gains table is overwritten.

The motivation for this routine arose from BIMA datasets in which the

calibrator is weak so that conventional amplitude calibration was not

very successful. This was compounded by old BIMA ant3 being blown about

by the wind. The source, on the other hand, was strong but time

variable, hence not susceptible to self-cal. From observations of

strong planets before and after it was observed that the amplitude

scale was reasonably stable, and certainly more stable than implied

by the phase-calibrator data, so that only phase calibration really needs

to be applied to the source. This program allows the amplitude scale

to be forced to the desired values (in this case, obtained from the

planet observations) without changing the phase calibration.

This routine is specific to CARMA, hardcoded for 15 antennae.

%A vis

The input visibility file, containing the gain file to list/massage

%A options

amp      List the amplitude gains: default option

The mean, median and rms (about the mean) are reported.

phase    List the phase corrections.

complex  List complex gains for 10 antennas only (2 lines per soln)

all      List all complex gains (one line per antenna per solution

for all antennas; lots of output, better than

options=complex if you want to grep one one antenna.)

replace  Replace the amplitude gains with the list supplied

Unless OPTIONS=FORCE is also set, only antennas with

non-zero values in the list are affected

so if jyperk is not set, nothing happens. Phases are

preserved unless options=zerophas is also specified

force    If set, then all values in jyperk are enforced when

doing a replace, even if they (or the initial gains)

are zero

limit    Impose an upper limit on the amplitude gains using the

list specified in jyperk

multiply Multiply existing sqrt(Jy/K) values in a gains table by

the list supplied in the jyperk variable. Only antennas

corresponding to nonzero jyperk elements are changed.

No effect on phases.

zerophas Zero all phase corrections (no antenna selection method)

addphase Correct gains by antenna based phases supplied in the

jyperk array, units degrees. Note that these phases

are added to the existing phase gains, except when

OPTIONS=ZEROPHAS is also used.

clip     Set to zero all gains outside range jyperk(1),jyperk(2)

Useful for pseudo-flagging of bad data, e.g.,

gplist vis=dummy options=clip jyperk=0.5,2.0

effectively flags data with gains outside 0.5-2 (default

range). However, data are not really flagged. The next

option is an alternative "flagging" option.

sigclip  Set to zero all gains more than jyperk(1)*rms away from

median on each antenna

Use options=replace,zerophas with suitable jyperk list to

both set amp scale and zero phases (the two steps are

carried out sequentially with the amplitudes being set first)

Use options=addphase to add phases to the existing ones.

Use options=addphase,force to replace phases.

%A ants

If given, this is the list of ants you want to see displayed.

Useful if you want a less wide screen display.

By default all are displayed, within limits.

%A jyperk

Array of 15 numbers (1 per antenna) giving the Jy-per-K values.

Array elements default to zero so you don't have to give 15 numbers.

Action will only be taken for antennas corresponding to nonzero

elements of jyperk.

Used for options=replace or options=multiply. For options=replace,

if any of the numbers are zero then the amp gains in the pre-existing

table will not be changed, so you can change the gains on a single

antennna without changing the others by setting all the other

values to zero. However, be aware that your one bad antenna will

have affected the solutions for the other antennas as well.

For options=multiply, jyperk supplies a list of multiplication

factors (one per antenna) which will be used to multiply the

sqrt(Jy/K) amplitude gains in the existing table.