%N uvfmeas

%D Calculate the fluxes of a point source at the phase

%P jbs

%: uv analysis

%B

UVFMEAS plots averaged spectra of a visibility dataset. It is

used to measure the spectral behaviour of a continuum source,

and can perform high order polynomial fits to the data. In this

way, a flux density at a particular frequency can be more

accurately determined. Multiple sets at different frequencies

can be simultaneously plotted, allowing you to make a global

fit over a wider range of frequencies, which may be more accurate.

%A vis

The name of the input uv data sets. Several can be given (wild

cards are supported). No default.

%A select

The normal uv selection commands. The default is plot everything.

%A line

The normal uv linetype in the form:

line,nchan,start,width,step

The default is all channels (or all wide channels if there are no

spectral channels).

%A stokes

The Stokes/polarization types to be plotted. The default is to

plot those polarizations present in the input files. Stokes

parameters will not however be overplot on each other; only one

polarization will be plotted at a time.

%A hann

Hanning smoothing width (an odd integer).  Smoothing is

applied after averaging. Default is 1 (no Hanning smoothing).

%A offset

An offset (in arcsec) to shift the data. Positive values result in

the data center being shifted to the North and East. Two values

should be given, being the shift in the RA and DEC directions.

The default is 0,0 (i.e. no shift).

%A order

The order of the highest term in the polynomial fit this task

should make to the spectrum it plots.

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreviated to the minimum

needed to avoid ambiguity. Possible options are:

'nocal'       Do not apply the gains file. By default, UVSPEC

applies the gains file in copying the data.

'nopass'      Do not apply bandpass corrections. By default,

UVSPEC corrects for the bandpass shape if the

required information is available.

'nopol'       Do not apply polarization corrections. By default

UVSPEC corrects for polarization cross-talk.

'log'         Do the spectral fitting in log space. By default

the fitting is done with the raw values. The plots

will still show the raw values when this option

is used. This option will be overridden, and a

warning displayed, if there are negative raw

values that make it impossible to do a log fit.

'plotvec'     Use the vector average values for the plot and

the fit. By default, the scalar average values

are used.

'uvhist'      Plot the uvdistance vs spectrally-corrected

amplitudes after plotting the spectrum and fit.

This will also show the histogram fit to the

data used to estimate the usability of the source

as a calibrator.

'plotfit'     Plot a user-specified fit over the spectrum. The

coefficients of the fit are given with the fitp

parameter.

'machine'     Output the fit coefficients on a single line,

separated by spaces, suitable for parsing by

another program.

'mfflux'      Output the fit coefficients in a way that can be

input as the flux parameter in mfcal. Only really

works for order=1 (linear fit).

'malpha'      Output the alpha coefficients on a single line,

separated by spaces, suitable for parsing by

another program.

'reshist'     Plot a histogram of the normalised residuals

from the fit.

%A yrange

The min and max range along the y axis of the plots. The default

is to autoscale.

%A device

PGPLOT plot device/type. No default.

%A nxy

Number of plots in the x and y directions. The default is

determined from the number of plots that are requested.

%A log

Log file into which the averaged data are dumped. If the option

'uvhist' is specified, the log file will contain the bin values:

uvdist val Np

Otherwise, the log file will contain the spectral values and fit:

freq fluxdensity fluxfit

%A fitp

The coefficients of a fit that you would like this task to

overplot onto the spectrum. The coefficients must relate to the

same type of fit (ie. log space or raw values) as the main fit

would use.

%A feval

A frequency (in GHz) at which to evaluate the fit, and output

the flux density in Jy.