%N calmake

%D Create gcal file from observation of calibrator(s)

%P pjt

%: calibration

%B

CALMAKE is a MIRIAD task which creates a calibration dataset.

it is needed because the calibration programs do not work

directly on standard visibility files.

CALMAKE reads in dataset(s) of amplitude/phase calibration source(s),

and creates a calibration dataset, called a ``gcal'' file.

The data within the gcal file is taken from either the wide

band correlation channels or some averaged value from the narrow

band channels.

%A vis

Input visibility dataset(s) to be used for calibration.

No default.

%A gcal

The output gain calibration dataset.

No default.

%A line

The ``line'' parameter determines the channels that are to be processed

from a uv data-set. The parameter value consists of a string

followed by up to four numbers. Defaults are used for any missing

trailing portion of the parameter value.

A uv data-set can have correlations from either (or both) a spectral

or a wideband (continuum) correlator. Both the spectral and wideband

correlators produce multiple correlations (channels). The string

part of the line parameter is used to select the spectral or wideband

channels. It may be one of:

"channel"   Spectral channels.

"wide"      Wideband (continuum) channels.

"velocity"  Spectral channels, resampled at equal increments in

velocity (using the radio definition). The resampling

involves a weighted average of the spectral channels.

This is useful if the source was not Doppler tracked

correctly.

"felocity"  Similar to "velocity", except that the parameters are

given using the optical velocity definition.

Generally the default is "channel" if the data-set has spectral

channel data, and "wide" otherwise.

The four numbers that accompany the string give:

nchan, start, width, step

These four numbers specify which channels to select from the input

dataset and how to combine them to produce the output channels that

the Miriad task will work on.

nchan   is the number of output channels produced. Generally it

defaults to the number of input channels.

start   is the first channel from input dataset that is to be used.

It defaults to 1 (i.e. first channel).

width   gives the number of input channels to average together to

produce a single output channel. It defaults to 1.

step    gives the increment between selected input channels. It

defaults to the value of "width".

For "velocity" linetype, the start, width and step parameters are

given in km/s. The output channels are centered on velocities:

start, start+step, start+2*step, etc.

The `line' parameter interacts with the "select=window" selection

for "channel" and "velocity"/"felocity" linetypes. See the help on

select for more information.

For example:

line=channel,10

selects 10 output channels, being input spectral channels 1 to 10.

Similarly

line=channel,10,8,1,2

selects 10 output channels, starting at input channel 8, and skipping

every second input channel, whereas

line=channel,10,8,2,2

selects 10 output channels, again starting at input channel 8, but

each of the output channels consists of the average of two of the

input channels. Finally

line=velocity,10,-10,1,1

resamples the spectral data in velocity, to give 10 channels of width

1 km/s. The channels are centered at -10.0,-9.0,-8.0, etc, km/s.

Default: ``wide,2,1,1,1'', i.e. the digital wideband data.

Note that the number of channels MUST be 2.

%A select

This keyword selects the subset of the visibility data to be

processed. There are a number of subcommands, which may be

abbreviated. Each may be prefixed with a plus or minus sign to

indicate using or discarding the selected data. Many subcommands

can be given (separate them by commas).

Examples:

'select=time(03OCT30:05:42:08,03OCT30:05:43:09),ant(7)(2)'

'select=-auto,-ant(7,8)'

'select=-ant(7,8),vis(1,15),vis(31,45),vis(301,1000)'

'select=-ant(7,8),-ant(1)(4,6),-ant(2)(4,5,6),-ant(3)(4,5),-ant(4)(5,6),-ant(5)(6)'

See also documentation for subroutine selprobe(), which may very well differ

slightly from this document.

Subcommands include:

time(t1,t2)

Select data between times t1 and t2 (UT). Times are in the format:

yymmmdd.fff

or

yymmmdd:hh:mm:ss.s

Various abbreviations are possible. If no date part is given, then

the time matchs all data (regardless of its date) for the given

time of day. e.g. time(03OCT30:05:41:08,03OCT30:05:43:09)

antennae(a1,a2,...)(b1,b2...)

Select all baselines pairs formed between first and second list of

antennas. The second list is optional and defaults to all antennas.

uvrange(uvmin,uvmax)

Select visibilities with uv radius between uvmin and uvmax (in kilo

wavelenghts). If only one value is given, uvmin is taken as zero.

uvnrange(uvmin,uvmax)

Select visibilities with uv radius between uvmin and uvmax (in

nanoseconds). If only one value is given, uvmin is taken as zero.

visibility(n1,n2)

Select visibilities numbered n1 to n2 inclusive.

increment(inc)

Select every inc'th visibility.

ra(r1,r2)

Select visibilities whose RA is in the range r1 to r2. RAs are given

in the format

hh:mm:ss

or

hh.hhh

Various abbreviations are possible.

dec(d1,d2)

Select visibilites whose DEC is in the range d1 to d2. Declinations

are given in the format

dd:mm:ss

or

dd.ddd

Various abbreviations are possible.

ha(h1,h2)

Selects on hour angle (values in decimal hours or hh:mm:ss)

lst(lst1,lst2)

Selects on LST (values in decimal hours or hh:mm:ss)

elevation(el1,el2)

Selects on elevation (values in degrees)

dra(p1,p2)

Select visibilities for which the RA of the pointing center is offset

from the main pointing center by between p1 and p2 arcseconds.

ddec(p1,p2)

Select visibilities for which the DEC of the pointing center is offset

from the main pointing center by between p1 and p2 arcseconds.

dazim(p1,p2)

Select visibilities for which the DAZIM offset is between p1 and p2 arcseconds.

delev(p1,p2)

Select visibilities for which the DELEV offset is between p1 and p2 arcseconds.

pointing(p1,p2)

Select visibilities with the rms pointing error in the range p1 to p2

arcseconds. If only one number is given, p1 is taken as 0.

polarization(p1,p2,p3,...)

Select visibilities on the basis of their polarization/Stokes parameter.

p1,p2,p3,... can be selected from "i,q,u,v,xx,yy,xy,yx,rr,ll,rl,lr"

Conversion between polarizations Stokes parameters (e.g. convert

linears to Stokes)  is not performed by this mechanism (see keyword stokes).

source(SRCNAM1,SRCNAM2,...)

Select correlations from the particular source. Several sources

can be given. An asterisk in the source name is treated as a

wildcard. Note that the sourcename match is done IGNORING CASE.

purpose(TYPE)

Select based on the intent/purpose of the observation. Introduced in

CARMA, where the letters B (bandpass), F (fluxcal), G (gaincal),

P (polarization), R (radio pointing), S (science) and O (other)

are used.

Multiple letters from this set can be chooses. No wildcards allowed.

seeing(r1,r2)

Select on seeing monitor rms path length (in microns). The UV variable used

is smonrms for ATCA, rmspath for CARMA

frequency(f1,f2)

Select correlations, where the sky frequency of the first correlation

is in the range f1 to f2 (in GHz). If only a single frequency is

given, then correlations are selected if the first correlation

is within 1% of the given frequency.  Note this selects the whole

record.  This may include several spectral-windows whose frequency

is quite different from the first correlation in the first window.

amplitude(amplo,amphi)

Select any correlation, where the amplitude is between "amplo" and

"amphi". If only one value is given, then "amphi" is assumed to be

infinite.

shadow(d)

Selects data that would be shadowed by an antenna of diameter "d" meters.

If "d" is zero, then the actual diameter of the antennas (if known) is used.

If some data is shadowed, it is advisable to use an antenna diameter value

greater than the physical antenna size (e.g. 20% larger).

See also the task CSFLAG for more finegrained approach for heterogeneous

arrays and dealing with sub arrays.

bin(b1,b2)

Select visibilities which have pulsar bin numbers in the range

b1 to b2 inclusive. If b2 is omitted, just pulsar bin b1 is

selected.

on(n)

This is used with single dish observations, anbd selects based

whether the "on" variable in the data is on (1) or off (0).

auto

This is used with files that contain a mix of autocorrelations and

crosscorrelations. This selects just the autocorrelation data.

window(w1,w2,...)

Select by spectral window (IF band in AIPS terminology). See

the help on window for more information.

or

The list of subcommands before and after the 'or' are "ored"

together.

and

Default ???

Default: all data from the input datasets.

%A auto

A logical, signalling if breakpoints are to be set automatically

when the source name changes.

Data before and after a breakpoint are then fit independently by

separate polynomials in subsequent fitting programs.

Also, data observed between the calibrators on either side of the

breakpoint will be flagged bad in calapply if flagbad=true,true.

Default: False (does not set breakpoints automatically).

%A fluxes

List of sources and associated fluxes in Jy to be used for

calibration. The format is name of the source, followed by

its flux, and optionally more sources, e.g.

fluxes=3c273,10.3,3c84,12.3,bllac,2.2

Currently no frequency dependency can be specified.

%A fluxtab

Name of the calibrators flux table file. If no name provided,

the system default (MIRCAT/cals.fluxes) is taken. The user

can also supply fluxes by hand (see keyword fluxes= above) in

which case the flux table derived values are ignored.

Note that reported frequencies of 0.0 means the flux was

determined from interpolation accross different frequencies.