%N uvredo

%D Recompute various derived quantitues in a uv data-set

%P rjs

%: uv analysis

%B

UVREDO recomputes various derived quantities in a uv data-set.

This may be needed if the derived quantities are wrong for some

reason, or if there is a need to change a coordinate system.

For example uvredo can recompute velocity information, thus

allowing conversion between different velocity rest frames.

A patch for handling SMA data has been added into UVREDO in

consideration of the fact that the sky frequency given in the

archived SMA data has been corrected for a part of the tracked

Doppler velocity (the diurnal term and a part of the annual term).

The SMA patch calculates the proper residual Doppler velocity for

each of the data records corresponding to the "corrected sky

frequency".

NOTE: The recomputation relies on a number of parameters in the

data-set, such as "time". It is assumed that this information is

correct. Uv variables that are important are:

For velocity: time and latitu,longitu (or telescop is these are

missing).

%A vis

The name of the input uv data set. No default.

%A select

The normal uv selection commands. The default is to select everything.

%A line

The normal uv linetype in the form:

line,nchan,start,width,step

The default is all channels. Note that if there are multiple

spectral windows, the fitting process is performed on each window

separately.

%A stokes

The polarization/Stokes parameters to be copied to the output

data-set. The default is to copy the polarisation/Stokes parameters

unchanged.

%A out

The name of the output uv data-set. There is no default name.

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreviated to the minimum

needed to avoid ambiguity. Possible options are:

'velocity'    Recompute velocity information.

'chi'         Recompute parallactic angle information.

The following options can be used to turn off calibration corrections.

The default is to apply any calibration present.

'nocal'       Do not apply the gains table.

'nopass'      Do not apply bandpass corrections.

'nopol'       Do not apply polarizatiopn corrections.

'smaveldop'   Recompute the veldop for SMA data.

Keyword: dopsour must be given.

%A dopsour

If options=smaveldop, this gives the source name, on which the Doppler

motions were tracked by the SMA online system.

dopsour is case-sensitive.

%A doptime

If options=smaveldop, this gives the UT time to calculate the annual

term as a reference value. The offset from this reference value in the

annual term has been used in the SMA online correction to the Doppler

tracked sky frequency in addition to the diurnal term. The keyword

doptime is in the format of [dd,hh,mm,ss.s; 4 values]. The default is

null. If doptime is null, UVREDO reads the residual Doppler velocity

(veldop) of the Doppler tracked source and recomputes the residual

Doppler velocity for each of the selected data records. The residual

Doppler velocity is then stored into the header variable "veldop"

corresponding to the "corrected sky frequency" ("sfreq").

%A velocity

If options=velocity or options=smaveldop, this gives the rest frame

of the output data-set. Possible values are 'observatory', 'lsr' and

'barycentric'. The default is the rest frame of the input data-set.