%N bldelay

%D Derive atmospheric delay differences from baseline phases.

%P jwl

%: uv calibration

%B

BLDELAY is a Miriad program that reads baseline phases and

derives estimates for the atmospheric delay. The visibility

data should have been calibrated to give zero phases in

all windows and channels at the start of the data. There is

no averaging done. If this is required it should be done in

another Miriad task.

%A vis

The input visibility file. No default.

%A out

The file containing the derived delays.

%A options

The options are

mean        (default) estimate the delay from the mean phase

across the band divided by the mean frequency,

and zero phase at zero frequency

slope       estimate the delay from the slope across all the

bands or channels

track       track phases in time to resolve wraps

%A select

This selects the data to be processed, using the standard uvselect

format. Default is all data.

%A line

Specify whether to use the wideband ("wide") or channel

("channel") data to derive the delays.

%A wraps

Number of 2*pi steps to search for the best delay fit. The

default is 3 (to search +/- 3*pi)