%N warpfit

%D Non linear Least Squares Fitting for Parablic Panels.

%P mchw

%: misc, utility

%B

WARPFIT determines the RMS deviation for a paraboloid

with a focal length of 100.8 inches for a maximum of 600 data points.

The rotation angle is set to zero. Data should be free format:

x(1)    y(1)    z(1)

x(2)    y(2)    z(2)

.       .       .

x(i)    y(i)    z(i)

The y-axis runs along the panel centerline

The x-axis is perpendicular to the centerline

The z-axis is the depth of the machining measured from the (x,y) plane.

%A in

Input ascii file with up to 9 columns of data. No default.

%A npts

Select the first ``npts'' from the input file.

Default reads the whole file or 2000 which is the maximum allowed.

%A par

Initial estimates for parameters; rotations(degrees) and offsets(inches).

Exactly 6 numbers are needed here:  xrot,yrot,z0,warp,angle,y0

%A nfit

Number of paramters to fit. Remaining parameters are fixed.

E.g. nfit=3 would only fit xrot,yrot,z0, leaving warp,angle,y0 fixed.

Default: nfit=3

%A swapxy

Swap x and y axes. Y or N, Default=No.

%A log

Output logfile. It will contain the results, a histogram

with residuals and all selected measurements, the fit, and residuals.

Default: print results and histogram only to terminal.